DR. VISHWANATH KARAD MIT WORLD PEACE UNIVERSITY

School of Engineering & Technology

Department of Electrical & Electronics Engineering

Academic Year: 2025–26

CIRCUIT CONCLAVE HACKATHON 2026

BILL OF MATERIALS (BOM)

4×4 Register File with Read/Write/Clear

Harshvardhan Rathod (PRN: —————)  &  Alok Deep Verma (PRN: —————)  |  FY B.Tech E&CE  |  Div: 23  |  2025–26

Source: RAM-project-001.csv — KiCad BOM export. All ICs are standard 74-series TTL in DIP packages from Texas Instruments, sourced from the project custom library (RAM-project-001-library-footprint).

FULL COMPONENT LIST

| KiCad Ref | Part / Value | Specification | Qty | Purpose | Cost |
| --- | --- | --- | --- | --- | --- |
| A7408 | SN74LS08N | Quad AND Gate, DIP-14 | 1 | Write-enable clock gating |  |
| B074244 | SN74LS244N | Octal 3-State Buffer, DIP-20 | 1 | Tri-state read bus (Bus 0) |  |
| B174244 | SN74LS244N | Octal 3-State Buffer, DIP-20 | 1 | Tri-state read bus (Bus 1) |  |
| CC1 | Capacitor | Ceramic Disc, 0.1 µF 50V | 1 | VCC decoupling |  |
| CC2 | Capacitor | Ceramic Disc, 0.1 µF 50V | 1 | VCC decoupling |  |
| CNE555 | NE555P | 555 Timer, DIP-8 | 1 | 0.5 Hz astable clock |  |
| CR1 | Resistor | 10 kΩ ¼W Axial | 1 | 555 R1 — charge path |  |
| CR2 | Resistor | 68 kΩ ¼W Axial | 1 | 555 R2 — timing |  |
| D74139 | SN74LS139AN | Dual 2-to-4 Decoder, DIP-16 | 1 | 2-bit address decoding |  |
| D074175 | SN74LS175N | Quad D Flip-Flop, DIP-16 | 1 | 4-bit register — Row 0 |  |
| D174175 | SN74LS175N | Quad D Flip-Flop, DIP-16 | 1 | 4-bit register — Row 1 |  |
| D274175 | SN74LS175N | Quad D Flip-Flop, DIP-16 | 1 | 4-bit register — Row 2 |  |
| D374175 | SN74LS175N | Quad D Flip-Flop, DIP-16 | 1 | 4-bit register — Row 3 |  |
| DIS0–DIS3 | SW_100SP1T1B4M2QE | E-Switch SPST | 4 | Data input D0–D3 |  |
| AS0, AS1 | SW_100SP1T1B4M2QE | E-Switch SPST | 2 | Address input A0–A1 |  |
| N7402 | SN7402N | Quad NOR Gate, DIP-14 | 1 | Row-select clock gating |  |
| O7432 | SN7432N | Quad OR Gate, DIP-14 | 1 | Asynchronous clear logic |  |
| OL0–OL3 | CQY99 | LED 5mm T-1 3/4, LED_D5.0mm | 4 | Output bus bit indicators |  |
| OR0–OR3 | Resistor | 220 Ω ¼W Axial | 4 | LED current limiting |  |
| SWC1 | SW_100SP1T1B4M2QE | E-Switch momentary | 1 | Clear button (→ LOW) |  |
| SWW1 | SW_100SP1T1B4M2QE | E-Switch momentary | 1 | Write/RW button (→ HIGH) |  |

COMPONENT SUMMARY

| Category | Qty | KiCad Reference(s) | Cost |
| --- | --- | --- | --- |
| ICs (7 unique types) | 11 | A7408, B074244, B174244, CNE555, D74139, D074175–D374175, N7402, O7432 |  |
| Switches (E-Switch) | 8 | DIS0–DIS3, AS0, AS1, SWC1, SWW1 |  |
| LEDs | 4 | OL0–OL3 |  |
| Resistors | 6 | OR0–OR3, CR1, CR2 |  |
| Capacitors | 2 | CC1, CC2 |  |
| TOTAL | 31 | — |  |

Total: 31 components  |  Single +5V supply  |  PCB: KiCad RAM-project-001

PCB LAYOUT — KiCad RAM-project-001

The PCB layout below shows the placement of all 31 components across the memory, allocator, and clock sections.

Figure 1: 4 Row Memory Storage — KiCad PCB (RAM-project-001). Designer: Harshvardhan M Rathod | Teammate: Alok Deep Verma

COMPONENT NOTES

1. Integrated Circuits

All ICs are standard 74-series TTL in DIP packages from Texas Instruments. The SN74LS175N (×4) forms the 4-bit register array. The SN74LS139AN decodes the 2-bit address. The SN74LS244N (×2) provides tri-state bus buffering. NOR (SN7402N) and AND (SN74LS08N) gates form the clock-gating logic. OR gates (SN7432N) implement the asynchronous clear network.

2. Switches

All switches use the E-Switch SW_100SP1T1B4M2QE footprint. Data and address inputs use SPST variants (one per bit); Clear and Write use momentary variants. No separate clock button is placed — clock is generated automatically by the NE555P astable circuit at 0.5 Hz.

3. LEDs & Passives

Four CQY99 LEDs (OL0–OL3) with 220 Ω current-limiting resistors display the output bus. The NE555P timing network uses CR1=10 kΩ (R1), CR2=68 kΩ (R2), and a 20 µF electrolytic timing capacitor (add as polarised capacitor CP if not already in schematic). CC1, CC2 are 0.1 µF ceramic disc VCC decoupling capacitors.

4. Cost Column

The Cost column has been left blank as component prices are subject to change. Please refer to current pricing from authorised distributors (e.g., DigiKey, Mouser, Element14) or local suppliers at the time of procurement.

| Submission Date: __________________ Verified by (Faculty Mentor): __________________________ | Faculty Mentor Signature: __________________________ Team prismlabs Signature: __________________________ |
| --- | --- |